ATV DRIP TORCH PRE-USE CHECKLIST

□ ATV personnel have proper PPE.

□ ATV is properly equipped and in serviceable condition.

- Fire extinguisher 2A 10 B:C

- First aid kit

- Tool kit

- Hand tool

□ Frame and Mounting System

- Mounting plates are firmly attached to the ATV rack.

- ATV rack is securely attached, attachment hardware is tight.

- Check torch frame welds for cracks or breaks.

- Ejection system adjusted with holding clamps in place and secure.

□ Tanks - Fuel, Propane, Nitrogen

- Tanks are full and extras available for change out.

- Tanks are secured and main tank is bolted down with no loose bolts.

- Fuel (torch) tank cap is in place with serviceable gasket.

□ Wand

- Break-away lock clamp adjusted to proper tension.

- Wand length adjusted and clevis pin in place.

- Propane igniter works properly and is secured to wand.

- Trigger lock will hold trigger in on position.

- Discharge check valve (at wand tip) stops flow of fuel when supply line is closed.

□ Gauges and Hoses

- All hoses and fittings attached and connections are tight, look for kinks or deformed hoses.

- Gauges function properly and are easy to read.

- Nitrogen regulator gauges function properly and pressure is adjusted for correct flow.

- Pressure relief valve is clean and operation checked manually (pull stem), over-pressurized to check automatic operation.  Valve should open automatically at 12-15 PSI.  DO NOT operate unit if valve fails to open in this pressure range.

□ All valves work properly.

- Main supply fuel valve (under tank)

- Fuel On-Off valve

- Fuel metering valve

- Propane On-Off valve