| U.S. Department of the Interior Bureau of Land Management JOB HAZARD ANALYSIS | Date: | New: Revised: |
| --- | --- | --- |
|  | Page 1 of 3 ATV DRIP TORCH OPERATIONS | Reviewed by: (Safety Mgr) |
| Field Office/Work Group | Supervisor: | Qual, Trng, Experience Required: Yes |
| This JHA must be reviewed, approved, and signed by the Agency Administrator: Name: Title: Date: |  |  |

| BASIC JOB STEPS | POTENTIAL HAZARDS | SAFE JOB PROCEDURES |
| --- | --- | --- |
| 1. All Terrain Vehicle (ATV) Operation |  | - All ATV operators must be trained and authorized to operate ATVs prior to operating vehicle. - Supervisor ensures ATV safety training is current. |
|  | A. General operational hazards (includes dust, flying debris, low hanging branches, etc.). | - ATV operator must wear required Personal Protective Equipment (PPE) including DOT-approved helmet, goggles or face shield, leather gloves, 8” leather boots and long sleeve Nomex fire shirt and Nomex fire pants. - Maintain all brush guards in working condition. - Securely fasten helmet with chin strap. |
|  | B. ATV rollover. | - ATV operator should reduce speed before making turns in uneven terrain. - Apply proper weight shift techniques. |
|  | C. Technical terrain (obstacles, off camber terrain, steep climbs/descents, low friction surfaces, etc.). | - Wide variety of terrain conditions may be encountered. Operator should use caution and proper riding techniques for safe operation in these areas. - Apply proper weight shift techniques. - Maintain safe speeds, use only low gears when operating ATV off established roads and trails. |
|  | D. Cargo hauling. | - Do not exceed ATV rack or total load capacity. - Load ATV so the center of gravity is kept as low as possible. - Secure loads so they do not shift. |
|  | E. ATV Loading/Unloading. | - ATV operator should wear full PPE when loading/unloading ATVs from transport vehicles. - Follow SOP for loading/unloading ATVs. |
|  | F. Collision with other vehicles. | - Avoid riding on paved roads. - Use headlights when riding on range and forest roads. - Operate vehicle at safe speeds and near the right edge of the road. |
| 2. Fueling and Servicing ATV | A. Exposure to sparks. | - Use proper fuel containers. - Move away from hot areas of burn. - No smoking. |
|  | B. Eye or skin contamination by fuel. | - Wear gloves and goggles. |
|  | C. Slippery surfaces from spilled fuel. | - Make every effort to avoid fuel spills. - Clean up spills as soon as possible. |
|  | D. Burns from hot engine. | - Turn engine off before refueling. - Allow engine to cool before servicing or refueling. |
| 3. ATV Drip-Torch Operations | A. Intrinsic danger of using ATV mounted drip torch. | - ATV torch is to be operated under supervision of Firing Leader or Firing Boss. - Use only with trained ATV operators and RX support personnel. Recommend: Ignition operation be performed with two ATVs and operators, one ATV drip torch operator and second ATV operator for oversight. |
|  | B. Inspection/Maintenance. | - Conduct thorough inspection of ATV and torch. - Check torch mounting, plumbing connections and controls; assure all are in proper working order. |
|  | C. Close proximity to fire, intense heat, and erratic fire behavior. | - Full complement of fire PPE. - Maintain close supervision and communications. - Post lookouts. - Know escape routes and safety zones. - Avoid firing with the wind directly behind the ATV. - Fire into the wind or with a cross wind. - Provide ample distance between lines of fire. - Firing patterns should allow ample time to escape fire. - LCES. |
|  | D. Rough terrain, ground fuels, side hills and slopes. | - Operator should use caution and proper riding techniques for safe passage through these areas. - Keep the center of gravity as low as possible. - Be aware of rocks, logs, trees, etc. - Pre-scout routes. |
|  | E. Inadvertent ignitions. | - Operator trained in ATV drip torch ignition procedures. - Develop pre-plan ignition points. - Check wand for correct operation on a regular basis. |
|  | F. ATV drip torch catches fire. | - Keep ATV engine running in neutral and brakes applied - Operator will use extinguisher to put out fire. - If unable to extinguish fire operator will use quick release mechanism to dump torch and drive away. - Notify appropriate supervisor of problem. |
| 4. Using ATV During Prescribed Firing Operations | A. Vehicle and operator exposed to fire during operations. | - Fire shelter will be worn by operator and not fastened to the ATV. - A hand tool will be carried on all ATVs used for firing operations. - Firing must not limit operator’s ability to keep both hands on the ATV controls when moving. - Protective skid plates must be on the ATV to prevent stumps or other debris from hanging up ATV. - The operator will recon and scout burn area before firing. In thick fuels, operator will ride into the area and fire on the way out. |
|  | B. ATV stuck or hung up within burn unit. | - Shut off fuel to drip torch. - Contact Burn Boss immediately via radio. - Identify escape routes and safety zones. Time permitting, try to free ATV. - Use hand tool to clear vegetation to mineral soil around ATV if time permits. |